VPU WRAPPER

API Specification

Date: June. 5, 2019

Freescale Semiconductor, Shanghai Software Development Center
UnitA, 20F, 560#, SongTao Road, Pu Dong New District

Shanghai, 201203, P.R of China

Revision History

| Version | Date | Revised By | Description of Changes |
| --- | --- | --- | --- |
| 1.0 | 06/05/2019 | Qi Hou | Initial version. Based on vpu wrapper code. |

Contents

1. Introduction	6

1.1. In this document	6

2. Data Types	6

2.1. Handle of vpu encoder and decoder	6

2.2. Enumerations	6

2.2.1. VpuEncRetCode/VpuDecRetCode	6

2.2.2. VpuDecBufRetCode	7

2.2.3. VpuEncBufRetCode	7

2.2.4. VpuDecCapability	7

2.2.5. VpuDecConfig	7

2.2.6. VpuEncConfig	8

2.2.7. VpuMemType	8

2.2.8. VpuDecErrInfo	8

2.2.9. VpuPicType	9

2.2.10. VpuFieldType	9

2.2.11. VpuType	9

2.2.12. VpuCodStd	9

2.2.13. VpuDecSkipMode	10

2.2.14. VpuDecInputType	10

2.2.15. VpuColorFormat	11

2.2.16. VpuEncMirrorDirection	11

2.2.17. VpuMemDescType	11

2.3. Structures	12

2.3.1. VpuMemSubBlockInfo	12

2.3.2. VpuMemInfo	12

2.3.3. VpuVersionInfo	12

2.3.4. VpuWrapperVersionInfo	12

2.3.5. VpuDecOpenParam	13

2.3.6. VpuFrameBuffer	13

2.3.7. VpuRect	14

2.3.8. VpuHDR10Meta	14

2.3.9. VpuColourDesc	14

2.3.10. VpuChromaLocInfo	14

2.3.11. VpuDecInitInfo	14

2.3.12. VpuFrameExtInfo	15

2.3.13. VpuDecOutFrameInfo	15

2.3.14. VpuCodecData	16

2.3.15. VpuBufferNode	16

2.3.16. VpuMemDesc	16

2.3.17. VpuDecFrameLengthInfo	16

2.3.18. VpuEncInitInfo	17

2.3.19. VpuEncOpenParamSimp	17

2.3.20. VpuEncSliceMode	17

2.3.21. VpuEncOpenParam	17

2.3.22. VpuEncEncParam	18

3. Decoder API Functions	19

3.1. Decoder Open and Close	19

3.1.1. VPU_DecGetVersionInfo	19

3.1.2. VPU_DecGetWrapperVersionInfo	19

3.1.3. VPU_DecGetInitialInfo	20

3.1.4. VPU_DecConfig	20

3.1.5. VPU_DecOpen	20

3.1.6. VPU_DecGetCapability	21

3.1.7. VPU_DecDisCapability	21

3.1.8. VPU_DecGetErrInfo	22

3.1.9. VPU_DecGetNumAvailableFrameBuffers	22

3.1.10. VPU_DecLoad	22

3.1.11. VPU_DecUnLoad	23

3.1.12. VPU_DecReset	23

3.1.13. VPU_DecClose	23

3.1.14. VPU_DecFlushAll	23

3.2. Decode	24

3.2.1. VPU_DecDecodeBuf	24

3.2.2. VPU_DecGetOutputFrame	24

3.2.3. VPU_DecGetConsumedFrameInfo	25

3.2.4. VPU_DecOutFrameDisplayed	25

3.3. Memory Query and Free	25

3.3.1. VPU_DecQueryMem	25

3.3.2. VPU_DecGetMem	26

3.3.3. VPU_DecFreeMem	26

3.3.4. VPU_DecRegisterFrameBuffer	26

4. Encoder API Functions	27

4.1. Encoder Open and Close	27

4.1.1. VPU_EncGetVersionInfo	27

4.1.2. VPU_EncGetWrapperVersionInfo	27

4.1.3. VPU_EncGetInitialInfo	27

4.1.4. VPU_EncConfig	28

4.1.5. VPU_EncOpen	28

4.1.6. VPU_EncOpenSimp	29

4.1.7. VPU_EncLoad	29

4.1.8. VPU_EncUnLoad	29

4.1.9. VPU_EncReset	29

4.1.10. VPU_EncClose	30

4.2. Encode	30

4.2.1. VPU_EncEncodeFrame	30

4.3. Memory Query and Free	30

4.3.1. VPU_EncQueryMem	30

4.3.2. VPU_EncGetMem	31

4.3.3. VPU_EncFreeMem	31

4.3.4. VPU_EncRegisterFrameBuffer	31

5. API Calling Sequence	32

5.1. Decoding calling sequence	32

5.2. Encoding calling sequence	34

# Introduction

## In this document

Vpu wrapper library provides encoding and decoding functions of streams.

This document is the API specification for the vpu wrapper library. The calling sequence of the API functions is also explained.

The API is explicitly described in “vpu_wrapper.h”.

# Data Types

## Handle of vpu encoder and decoder

typedef void * VpuEncHandle;

typedef void * VpuDecHandle;

## Enumerations

### VpuEncRetCode/VpuDecRetCode

Specifies the return values for the encoder/decoder API functions.

| VpuEncRetCode/VpuDecRetCode value | Description |
| --- | --- |
| VPU_ENC_RET_SUCCESS/ VPU_DEC_RET_SUCCESS | Success. |
| VPU_ENC_RET_FAILURE/ VPU_DEC_RET_FAILURE | Fail. |
| VPU_ENC_RET_INVALID_PARAM/ VPU_DEC_RET_INVALID_PARAM | Parameter is invalid. |
| VPU_ENC_RET_INVALID_HANDLE/ VPU_DEC_RET_INVALID_HANDLE | Handle is invalid. |
| VPU_ENC_RET_INVALID_FRAME_BUFFER/ VPU_DEC_RET_INVALID_FRAME_BUFFER | Frame buffer is invalid. |
| VPU_ENC_RET_INSUFFICIENT_FRAME_BUFFERS/ VPU_DEC_RET_INSUFFICIENT_FRAME_BUFFERS | Frame buffers is insufficient. |
| VPU_ENC_RET_INVALID_STRIDE/ VPU_DEC_RET_INVALID_STRIDE | Stride is invalid. |
| VPU_ENC_RET_WRONG_CALL_SEQUENCE/ VPU_DEC_RET_WRONG_CALL_SEQUENCE | State of the object is not correct. |
| VPU_ENC_RET_FAILURE_TIMEOUT/ VPU_DEC_RET_FAILURE_TIMEOUT | The wait for HW/SW finish has timed out. |

### VpuDecBufRetCode

Specifies the data state of input and output frame when decoding.

| VpuDecBufRetCode value | Description |
| --- | --- |
| VPU_DEC_INPUT_NOT_USED | Input data has been consumed. |
| VPU_DEC_INPUT_USED | Input data hasn’t been consumed. |
| VPU_DEC_OUTPUT_EOS | Got eos. |
| VPU_DEC_OUTPUT_DIS | Got one frame to output. |
| VPU_DEC_OUTPUT_NODIS | Got no frame to output. |
| VPU_DEC_OUTPUT_REPEAT | One frame is output repeatly. |
| VPU_DEC_OUTPUT_DROPPED | Got one frame to drop. |
| VPU_DEC_OUTPUT_MOSAIC_DIS | Got one mosaic frame to output. |
| VPU_DEC_NO_ENOUGH_BUF | No enough buffer to hold for output. |
| VPU_DEC_NO_ENOUGH_INBUF | No enough input buffer. |
| VPU_DEC_INIT_OK | Initialization of decoding is ok. |
| VPU_DEC_SKIP | Skip to decode one frame. |
| VPU_DEC_ONE_FRM_CONSUMED | One frame has been decoded. |
| VPU_DEC_RESOLUTION_CHANGED | Resolution of the frame is changed. |
| VPU_DEC_FLUSH | Flush the decoder. |

### VpuEncBufRetCode

Specifies the data state of input and output frame when encoding.

| VpuEncBufRetCode value | Description |
| --- | --- |
| VPU_ENC_INPUT_NOT_USED | Input data has been consumed. |
| VPU_ENC_INPUT_USED | Input data hasn’t been consumed. |
| VPU_ENC_OUTPUT_SEQHEADER | Sequence header (for H.264: SPS/PPS). |
| VPU_ENC_OUTPUT_DIS | Got one frame to output. |
| VPU_ENC_OUTPUT_NODIS | Got no frame to output. |

### VpuDecCapability

Specifies the capability type of decoder.

| VpuDecCapability value | Description |
| --- | --- |
| VPU_DEC_CAP_FILEMODE = 0 | File mode is supported ? 0: not; 1: yes |
| VPU_DEC_CAP_TILE | Tile format is supported ? 0: not; 1: yes |
| VPU_DEC_CAP_FRAMESIZE | Reporting frame size ? 0: not; 1: yes |
| VPU_DEC_CAP_RESOLUTION_CHANGE | Resolution change notification ? 0: not; 1: yes |

### VpuDecConfig

Specifies the configuration types of decoder.

| VpuDecConfig value | Description |
| --- | --- |
| VPU_DEC_CONF_SKIPMODE = 0 | parameter value: VPU_DEC_SKIPNONE(default); VPU_DEC_SKIPPB; VPU_DEC_SKIPB; VPU_DEC_SKIPALL; VPU_DEC_ISEARCH. |
| VPU_DEC_CONF_INPUTTYPE | parameter value: VPU_DEC_IN_NOMAL: normal(default); VPU_DEC_IN_KICK: kick -- input data/size in VPU_DecDecodeBuf() will be ignored; VPU_DEC_IN_DRAIN: drain -- stream reach end, and input data/size in VPU_DecDecodeBuf() will be ignored. |
| VPU_DEC_CONF_BUFDELAY | For stream mode. The parameter represents buf size(unit: bytes), buffer size==0 indicate no any delay. |
| VPU_DEC_CONF_INIT_CNT_THRESHOLD | At seqinit stage. vpu will report error if data count reach the threshold. |
| VPU_DEC_CONF_ENABLE_TILED | Configure output frame to tiled after parsed sequence header and before register frame buffer. |

### VpuEncConfig

Specifies the configuration types of encoder.

| VpuEncConfig value | Description |
| --- | --- |
| VPU_ENC_CONF_NONE = 0 | None. |
| VPU_ENC_CONF_BIT_RATE | Set bit rate(unit: kbps). |
| VPU_ENC_CONF_INTRA_REFRESH | Intra refresh: minimum number of macroblocks to refresh in a frame. |
| VPU_ENC_CONF_ENA_SPSPPS_IDR | some muxers may ignore the sequence or config data, so SPS/PPS is needed for every IDR frame, including the first IDR. |
| VPU_ENC_CONF_RC_INTRA_QP | Intra qp value. |
| VPU_ENC_CONF_INTRA_REFRESH_MODE | Intra refresh mode: 0: normal; 1: cyclic. |

### VpuMemType

Specifies the memory type of vpu.

| VpuMemType value | Description |
| --- | --- |
| VPU_MEM_VIRT = 0 | 0 for virtual memory |
| VPU_MEM_PHY = 1 | 1 for physical continuous memory |

### VpuDecErrInfo

Specifies the type of error information when decode.

| VpuDecErrInfo value | Description |
| --- | --- |
| VPU_DEC_ERR_UNFOUND = 0 | None. |
| VPU_DEC_ERR_NOT_SUPPORTED | The profile/level/features/... outrange the vpu's capability. |
| VPU_DEC_ERR_CORRUPT | Some syntax errors are detected. |

### VpuPicType

Specifies the picture type of vpu.

| VpuPicType value | Description |
| --- | --- |
| VPU_I_PIC = 0 | I frame or I sclie(H.264). |
| VPU_P_PIC | P frame or P sclie(H.264) . |
| VPU_B_PIC | B frame or B sclie(H.264). |
| VPU_IDR_PIC | IDR frame(H.264). |
| VPU_BI_PIC | BI frame(VC1). |
| VPU_SKIP_PIC | Skipped frame(VC1). |
| VPU_UNKNOWN_PIC | Reserved. |

### VpuFieldType

Specifies the filed type of vpu.

| VpuFieldType value | Description |
| --- | --- |
| VPU_FIELD_NONE = 0 | Frame. |
| VPU_FIELD_TOP | Only top field. |
| VPU_FIELD_BOTTOM | Only bottom field. |
| VPU_FIELD_TB | Top field + Bottom field. |
| VPU_FIELD_BT | Bottom field + Top field. |
| VPU_FIELD_UNKNOWN | Reserved. |

### VpuType

Specifies the type of vpu.

typedef enum {

VPU_TYPE_UNKNOWN = 0,

VPU_TYPE_CHIPSMEDIA,

VPU_TYPE_MALONE,

VPU_TYPE_HANTRO,

} VpuType;

### VpuCodStd

Specifies the video type.

typedef enum {

VPU_V_MPEG4 = 0,

VPU_V_DIVX3,

VPU_V_DIVX4,

VPU_V_DIVX56,

VPU_V_XVID,

VPU_V_H263,

VPU_V_AVC,

VPU_V_AVC_MVC,

VPU_V_VC1,

VPU_V_VC1_AP,

VPU_V_MPEG2,

VPU_V_RV,

VPU_V_MJPG,

VPU_V_AVS,

VPU_V_VP8,

VPU_V_VP9,

VPU_V_HEVC,

VPU_V_SORENSON,

VPU_V_VP6,

VPU_V_WEBP,

} VpuCodStd;

### VpuDecSkipMode

Specifies the skip mode when decoding.

typedef enum {

VPU_DEC_SKIPNONE=0,

VPU_DEC_SKIPPB,

VPU_DEC_SKIPB,

VPU_DEC_SKIPALL,

VPU_DEC_ISEARCH,	      /*only decode IDR*/

}VpuDecSkipMode;

### VpuDecInputType

Specifies the input type of decoder.

typedef enum {

VPU_DEC_IN_NORMAL=0,

VPU_DEC_IN_KICK,

VPU_DEC_IN_DRAIN,

}VpuDecInputType;

### VpuColorFormat

Specifies the color format of video.

typedef enum

{

VPU_COLOR_420=0,

VPU_COLOR_422H=1,

VPU_COLOR_422V=2,

VPU_COLOR_444=3,

VPU_COLOR_400=4,

VPU_COLOR_422YUYV=13,

VPU_COLOR_422UYVY=14,

VPU_COLOR_ARGB8888=15,

VPU_COLOR_BGRA8888=16,

VPU_COLOR_RGB565=17,

VPU_COLOR_RGB555=18,

VPU_COLOR_BGR565=19,

}VpuColorFormat;

### VpuEncMirrorDirection

Specifies the mirror direction of encoder.

typedef enum {

VPU_ENC_MIRDIR_NONE,

VPU_ENC_MIRDIR_VER,

VPU_ENC_MIRDIR_HOR,

VPU_ENC_MIRDIR_HOR_VER

} VpuEncMirrorDirection;

### VpuMemDescType

Specifies the memory type.

typedef enum{

VPU_MEM_DESC_NORMAL = 0,

VPU_MEM_DESC_SECURE = 1,

}VpuMemDescType;

## Structures

### VpuMemSubBlockInfo

A struct that contains the information of a subblock.

| Members | Type | Description |
| --- | --- | --- |
| nAlignment | int | Alignment limitation. |
| nSize | int | Size of memory length. |
| MemType | VpuMemType | Flag to indicate Static, scratch or output data memory. |
| pVirtAddr | unsigned char * | Virtual address of the pointer to the base memory. |
| pPhyAddr | unsigned char * | Physical address of the pointer to the base memory. |
| nReserved[3] | int | Reserved for future extension. |

### VpuMemInfo

A struct that contains memory information of all subblocks.

| Members | Type | Description |
| --- | --- | --- |
| nSubBlockNum | int | Number of subblocks. |
| MemSubBlock[VPU_DEC_ MAX_NUM_MEM_REQS] | VpuMemSubBlockInfo | VpuMemSubBlockInfo struct that contains subblock information. |

### VpuVersionInfo

A struct that contains vpu version information.

| Members | Type | Description |
| --- | --- | --- |
| nFwMajor | int | Firmware major version. |
| nFwMinor | int | Firmware minor version. |
| nFwRelease | int | Firmware release version. |
| nFwCode | int | Firmware code version. |
| nLibMajor | int | Library major version. |
| nLibMinor | int | Library minor version. |
| nLibRelease | int | Library release version. |
| nReserved | int | Reserved for future extension. |

### VpuWrapperVersionInfo

A struct that contains vpu wrapper version information.

| Members | Type | Description |
| --- | --- | --- |
| nMajor | int | Major version. |
| nMinor | int | Minor version. |
| nRelease | int | Release version. |
| pBinary | char * | Version info specified by user. |
| nReserved[4] | int | Reserved for future extension. |

### VpuDecOpenParam

A struct that contains the input parameters of decoder.

| Members | Type | Description |
| --- | --- | --- |
| CodecFormat | VpuCodStd | Type of video. |
| nReorderEnable | int | Whether to enable record. |
| nChromaInterleave | int | Should be set to 1 when (nMapType!=0). |
| nMapType | int | Registered frame buffer type: 0--linear; 1--frame tile; 2--field tile. |
| nTiled2LinearEnable | int | Output frame(only valid when nMapType!=0) : 0--tile; 1--linear(not supported). |
| nPicWidth | int | Width of the decoded image. |
| nPicHeight | int | Height of the decoded image. |
| nEnableFileMode | int | Whether filemode is enabled. |
| nEnableVideoCompressor | int | Whether video compressor is enabed. |
| nPixelFormat | int | Output 10 bit or cut to 8 bit for Hantro G2. 0 for output 10 bit, 1 for cut to 8 bit. |
| nAdaptiveMode | int | Whether in adaptive mode. |
| pAppCxt | void * | Reserved for future application extension. |
| nSecureMode | int | Whether in secure mode. |
| nSecureBufferAllocSize | int | Allocated buffer size in secure mode. |

### VpuFrameBuffer

A struct that contains the information of frame buffer in vpu.

| Members | Type | Description |
| --- | --- | --- |
| nStrideY | unsigned int | Luma stride information. |
| nStrideC | unsigned int | Chroma stride information. |
| pbufY | unsigned char * | Physical assress of luma frame pointer or top field pointer. |
| pbufCb | unsigned char * | Physical assress of chroma frame pointer or top field pointer. |
| pbufCr | unsigned char * |  |
| pbufMvCol | unsigned char * |  |
| pbufY_tilebot | unsigned char * | For field tile: physical assress of luma bottom pointer. |
| pbufCb_tilebot | unsigned char * | For field tile: physical assress of chroma bottom pointer. |
| pbufVirtY | unsigned char * | Virtual address of luma frame pointer or top field pointer. |
| pbufVirtCb | unsigned char * | Virtual address of chroma frame pointer or top field pointer. |
| pbufVirtCr | unsigned char * |  |
| pbufVirtMvCol | unsigned char * |  |
| pbufVirtY_tilebot | unsigned char * | For field tile: virtual address of luma bottom pointer. |
| pbufVirtCb_tilebot | unsigned char * | For field tile: virtual address of chroma bottom pointer. |
| nReserved[5] | int | Reserved for future extension. |
| pPrivate | void * | Reserved for future special extension. |

### VpuRect

A struct that contains the image information.

| Members | Type | Description |
| --- | --- | --- |
| nLeft | unsigned int | Size of the image left. |
| nTop | unsigned int | Size of the image top. |
| nRight | unsigned int | Size of the image right. |
| nBottom | unsigned int | Size of the image bottom. |

### VpuHDR10Meta

A struct that contains the meta data of video hdr10.

| Members | Type | Description |
| --- | --- | --- |
| redPrimary[2] | unsigned int |  |
| greenPrimary[2] | unsigned int |  |
| bluePrimary[2] | unsigned int |  |
| whitePoint[2] | unsigned int |  |
| maxMasteringLuminance | unsigned int |  |
| minMasteringLuminance | unsigned int |  |
| maxContentLightLevel | unsigned int |  |
| maxFrameAverageLightLevel | unsigned int |  |

### VpuColourDesc

A struct that contains color description.

| Members | Type | Description |
| --- | --- | --- |
| colourPrimaries | unsigned int |  |
| transferCharacteristics | unsigned int |  |
| matrixCoeffs | unsigned int |  |
| fullRange | unsigned int |  |

### VpuChromaLocInfo

A struct that contains chroma information.

| Members | Type | Description |
| --- | --- | --- |
| chromaSampleLocTypeTopField | unsigned int |  |
| chromaSampleLocTypeBottomField | unsigned int |  |

### VpuDecInitInfo

A struct that contains the initial information of decoder.

| Members | Type | Description |
| --- | --- | --- |
| nPicWidth | int | Aligned width of image. |
| nPicHeight | int | Aligned height of image. |
| nFrameRateRes | int | Numerator of framerate. |
| nFrameRateDiv | int | Denominator of framerate. |
| PicCropRect | VpuRect | VpuRect struct that contains crop information of image. |
| nMinFrameBufferCount | int | Minimum frame buffer count in vpu. |
| nMjpgSourceFormat | int | Source color format of jpeg |
| nInterlace | int | Whether video is interlaced. |
| nQ16ShiftWidthDivHeightRatio | unsigned int | Fixed point for width/height: 1: 0x10000; 0.5: 0x8000;... |
| nConsumedByte | int | Reserved to record sequence length: value -1 indicate unknow. |
| nAddressAlignment | int | Address alignment for Y/Cb/Cr (unit: bytes). |
| nFrameSize | int | Hantro video decoder append DMV and compression table in pixel buffer. |
| nBitDepth | int | Bit depth of video. |
| nReserved[3] | int | Reserved for future extension. |
| pSpecialInfo | void * | Reserved for future special extension. |
| hasColorDesc | int | Whether has color description. |
| hasHdr10Meta | int | Whether has hdr10 meta data. |
| Hdr10Meta | VpuHDR10Meta | Hdr10Meta struct which contains hdr10 meta data. |
| ColourDesc | VpuColourDesc | ColourDesc struct which contains color description. |
| ChromaLocInfo | VpuChromaLocInfo | VpuChromaLocInfo struct which contains chroma information. |

### VpuFrameExtInfo

A struct that contains the extended information of frame.

| Members | Type | Description |
| --- | --- | --- |
| nFrmWidth | int | Width of image. |
| nFrmHeight | int | Height of image. |
| FrmCropRect | VpuRect | VpuRect struct that contains the crop information of image. |
| nQ16ShiftWidthDivHeightRatio | unsigned int | Fixed point for width/height: 1: 0x10000; 0.5: 0x8000;... |
| rfc_luma_offset | int | Luma offset. |
| rfc_chroma_offset | int | Chroma offset. |
| nReserved[7] | int | Reserved for future extension. |

### VpuDecOutFrameInfo

A struct that contains information of output frame when decoding.

| Members | Type | Description |
| --- | --- | --- |
| pDisplayFrameBuf | VpuFrameBuffer * | Pointer to VpuFrameBuffer struct which contains the display information of the frame. |
| ePicType | VpuPicType | Type of frame. |
| eFieldType | VpuFieldType | Field type of vpu. |
| nMVCViewID | int | Extended info: support dynamic resolution, ... |
| pExtInfo | VpuFrameExtInfo * | Luma offset. |
| nReserved[2] | int | Reserved for future extension. |
| pPrivate | void * | Reserved for future special extension. |

### VpuCodecData

A struct that contains the codec data information.

| Members | Type | Description |
| --- | --- | --- |
| pData | unsigned char * | Codec data virtual address. |
| nSize | unsigned int | Codec data length. |

### VpuBufferNode

A struct that contains the information of a buffer in vpu.

| Members | Type | Description |
| --- | --- | --- |
| pPhyAddr | unsigned char * | Buffer physical base address. |
| pVirAddr | unsigned char * | Buffer virtual base address. |
| nSize | unsigned int | Length of data. |
| sCodecData | VpuCodecData | VpuCodecData struct that contains codec data information. |
| nReserved[2] | int | Reserved for future extension. |
| pPrivate | void * | Reserved for future special extension |

### VpuMemDesc

A struct that contains the description of memory.

| Members | Type | Description |
| --- | --- | --- |
| nSize | int | Requested memory size. |
| pPhyAddr | unsigned long | Physical memory address allocated. |
| nCpuAddr | unsigned long | Cpu address for system free usage. |
| nVirtAddr | unsigned long | Virtual user space address. |
| nType | VpuMemDescType | Type of memory. |
| nReserved[3] | int | Reserved for future extension. |

### VpuDecFrameLengthInfo

A struct that contains the information of the frame length in vpu.

| Members | Type | Description |
| --- | --- | --- |
| pFrame | VpuFrameBuffer * | Point to the frame buffer which contains the information of frame buffer. |
| nStuffLength | int | Stuff data length ahead of frame. |
| nFrameLength | int | Length of the frame. |
| nReserved[5] | int | Reserved for recording other info. |

### VpuEncInitInfo

A struct that contains the initial information of encoder.

| Members | Type | Description |
| --- | --- | --- |
| nMinFrameBufferCount | int | Minimum frame buffer count in vpu. |
| nAddressAlignment | int | Address alignment for Y/Cb/Cr (unit: bytes). |
| eType | VpuType | Type of vpu. |

### VpuEncOpenParamSimp

A struct that contains the simple input parameters of decoder.

| Members | Type | Description |
| --- | --- | --- |
| eFormat | VpuCodStd | Type of video. |
| nPicWidth | int | Width of the encoded image. |
| nPicHeight | int | Height of the encoded image. |
| nRotAngle | int | Rotate angle of the image. |
| nFrameRate | int | The framerate of the output frame. |
| nBitRate | int | Bit rate of the output frame. |
| nGOPSize | int | Number of pictures in one GOP. |
| nIntraRefresh | int | Intra macro block numbers. |
| nIntraQP | int | Qp values, 0: auto, >0: qp value. |
| nChromaInterleave | int | Should be set to 1 when (nMapType!=0). |
| sMirror | VpuEncMirrorDirection | Mirror direction of the encoder. |
| nMapType | int | Frame buffer: 0--linear ; 1--frame tile; 2--field tile. |
| nLinear2TiledEnable | int | Valid when (nMapType!=0): 0--tile input; 1--yuv input. |
| eColorFormat | VpuColorFormat | Color format of video. |
| nIsAvcc | int | Used for H.264 data format, 0: byte stream ; 1: avcc format. |
| nReserved[3] | int | Reserved for future extension. |
| pAppCxt | void * | Reserved for future extension. |

### VpuEncSliceMode

A struct that contains the slice information.

| Members | Type | Description |
| --- | --- | --- |
| sliceMode | int | The mode of slice. |
| sliceSizeMode | int | The mode of slice size. |
| sliceSize | int | Set the size of a slice. |
| nReserved | int | Reserved for future extension. |

### VpuEncOpenParam

A struct that contains input parameters of decoder.

| Members | Type | Description |
| --- | --- | --- |
| eFormat | VpuCodStd | Type of video. |
| nPicWidth | int | Width of the encoded image. |
| nPicHeight | int | Height of the encoded image. |
| nRotAngle | int | Rotate angle of the image. |
| nFrameRate | int | The framerate of the output frame. |
| nBitRate | int | Bit rate of the output frame. |
| nGOPSize | int | Number of pictures in one GOP. |
| nChromaInterleave | int | Should be set to 1 when (nMapType!=0). |
| sMirror | VpuEncMirrorDirection | Mirror direction of the encoder. |
| nMapType | int | Frame buffer: 0--linear ; 1--frame tile; 2--field tile. |
| nLinear2TiledEnable | int | Valid when (nMapType!=0): 0--tile input; 1--yuv input. |
| eColorFormat | VpuColorFormat | Color format of video. |
| nUserQpMax | int | Maximum user qp value. |
| nUserQpMin | int | Minimum user qp value. |
| nUserQpMinEnable | int |  |
| nUserQpMaxEnable | int |  |
| nIntraRefresh | int | Intra macro block numbers. |
| nRcIntraQP | int | Qp values, 0: auto, >0: qp value. |
| nUserGamma | int |  |
| nRcIntervalMode | int | 0:normal, 1:frame_level, 2:slice_level, 3: user defined Mb_level. |
| nMbInterval | int | Use when RcintervalMode is 3. |
| nAvcIntra16x16OnlyModeEnable | int |  |
| sliceMode | VpuEncSliceMode | A pointer to VpuEncSliceMode struct. |
| nInitialDelay | int |  |
| nVbvBufferSize | int |  |
| union { VpuEncMp4Param mp4Param; VpuEncH263Param h263Param; VpuEncAvcParam avcParam; } VpuEncStdParam; |  |  |
| nMESearchRange | int | 3: 16x16, 2:32x16, 1:64x32, 0:128x64, H.263(Short Header : always 3. |
| nMEUseZeroPmv | int | 0: PMV_ENABLE, 1: PMV_DISABLE. |
| nIntraCostWeight | int | Additional weight of Intra Cost for mode decision to reduce Intra MB density. |
| nIsAvcc | int | Used for H.264 data format, 0: byte stream ; 1: avcc format. |
| nReserved[8] | int | Reserved for future extension. |
| pAppCxt | void * | Reserved for future extension. |

### VpuEncEncParam

A struct that contains the input and output parameters of encoder.

| Members | Type | Description |
| --- | --- | --- |
| eFormat | VpuCodStd | Type of video. |
| nPicWidth | int | Width of the encoded image. |
| nPicHeight | int | Height of the encoded image. |
| nFrameRate | int | Framerate of the output frame. |
| nQuantParam | int | Quant parameter of the frame |
| nInPhyInput | unsigned long | Input buffer physical address. |
| nInVirtInput | unsigned long | Input buffer virtual address. |
| nInInputSize | int | Input buffer size. |
| nInPhyOutput | unsigned long | Output buffer physical address. |
| nInVirtOutput | unsigned long | Output buffer virtual address. |
| nInOutputBufLen | unsigned int | The left buffer size. |
| nForceIPicture | int | Whether to force the current frame as key frame. |
| nSkipPicture | int | Whether to skip current frame. |
| nEnableAutoSkip | int | Whether to enable auto skip. |
| eOutRetCode | VpuEncBufRetCode | Output state after encoding. |
| nOutOutputSize | int | Output buffer size. |
| pInFrame | VpuFrameBuffer * | Pointer to VpuFrameBuffer struct that contains the input frame information. |
| nReserved[2] | int | Reserved for future extension. |
| pPrivate | void * | Reserved for future extension. |

# Decoder API Functions

## Decoder Open and Close

### VPU_DecGetVersionInfo

VpuDecRetCode VPU_DecGetVersionInfo (

VpuVersionInfo 		* pOutVerInfo)

Description:

Function to get the vpulib and firmware version.

Arguments

pOutVerInfo [out]          	Pointer to VpuVersionInfo struct where output parameters will be saved.

Return value

VPU_DEC_RET_SUCCESS

### VPU_DecGetWrapperVersionInfo

VpuDecRetCode VPU_DecGetWrapperVersionInfo (

VpuWrapperVersionInfo 	* pOutVerInfo)

Description:

Function to get the vpu wrapper version.

Arguments

pOutVerInfo [out]   	Pointer to VpuWrapperVersionInfo struct where the output parameters

will be saved.

Return value

VPU_DEC_RET_SUCCESS

### VPU_DecGetInitialInfo

VpuDecRetCode VPU_DecGetInitialInfo (

VpuDecHandle 		InHandle,

VpuDecInitInfo 		* pOutInitInfo)

Description:

Function to get the initial info.

Arguments:

InHandle [in/out]  	Handle of vpu decoder.

pOutInitInfo [out] 	Pointer to VpuEncInitInfo struct where initial infomation is stored.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecConfig

VpuDecRetCode VPU_DecConfig (

VpuDecHandle 		InHandle,

VpuDecConfig 		InDecConf,

void			* pInParam)

Desciption:

Function to set corresponding parameters of decoder according to InDecConf type.

Arguments:

InDecConf [in] 		Type of configuration to be set.

pInParam [in]		Value that used to set the decoder according to InDecConf type.

InHandle [out]		Handle of vpu decoder.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecOpen

VpuDecRetCode VPU_DecOpen (

VpuDecHandle 		* pOutHandle,

VpuDecOpenParam 	* pInParam

VpuMemInfo		* pInMemInfo)

Description:

Function to open new vpu handle.

Arguments:

pInParam [in]	Pointer to VpuDecOpenParam struct which contains input parameters of the decoder.

pInMemInfo [in]		Pointer to VpuMemInfo struct which contains the memory information.

pOutHandle [out] 	Handle of vpu decoder.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecGetCapability

VpuDecRetCode VPU_DecGetCapability (

VpuDecHandle 		InHandle,

VpuDecCapability 	eInCapability,

int			* pOutCapbility)

Description:

Function to get capability of the input eInCapability. If eInCapability is supported, pOutCapbility will be set to 1.

Arguments:

InHandle [in]  		Handle of vpu decoder.

eInCapability [in] 	Type of configuration to be set.

pOutCapbility [out] 	Set to be 1 if eInCapability type is supported.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecDisCapability

VpuDecRetCode VPU_DecDisCapability (

VpuDecHandle 		InHandle,

VpuDecCapability 	eInCapability)

Description:

Function to get capability of the input eInCapability. If eInCapability is supported, pOutCapbility will be set to 1.

Arguments:

InHandle [in/out]  	Handle of vpu decoder, corresponding parameters will be set according to eInCapability type.

eInCapability [in] 	Type of configuration to be set.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecGetErrInfo

VpuDecRetCode VPU_DecGetErrInfo (

VpuDecHandle 		InHandle,

VpuDecErrInfo		* pErrInfo)

Description:

Function to get the error information.

Arguments:

InHandle [in]  		Handle of vpu decoder.

pErrInfo [out] 	Pointer to VpuDecErrInfo enumeration where contains type of error information.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecGetNumAvailableFrameBuffers

VpuDecRetCode VPU_DecGetNumAvailableFrameBuffers (

VpuDecHandle 		InHandle,

int			* pOutBufNum)

Description:

Function to get the number of available frame buffers.

Arguments:

InHandle [in]  		Handle of vpu decoder.

pOutBufNum [out] 	Number of the available frame buffers.

Return value:

VPU_DEC_RET_SUCCESS

### VPU_DecLoad

VpuDecRetCode VPU_DecLoad ()

Description:

Function to parse the log level, load vpu.

Arguments:

None.

Return value:

VPU_DEC_RET_SUCCESS

### VPU_DecUnLoad

VpuDecRetCode VPU_DecUnLoad ()

Description:

Function to unload vpu.

Arguments:

None.

Return value:

VPU_DEC_RET_SUCCESS

### VPU_DecReset

VpuDecRetCode VPU_DecReset (

VpuDecHandle 		InHandle)

Description:

Function to reset the handle of vpu deocder.

Arguments:

InHandle [in] 		Handle of vpu decoder.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecClose

VpuDecRetCode VPU_DecClose (

VpuDecHandle 		InHandle)

Description:

Function to close the decoder.

Arguments:

InHandle [in] 		Handle of vpu decoder.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecFlushAll

VpuDecRetCode VPU_DecFlushAll (

VpuDecHandle 		InHandle)

Description:

Function to flush the decoder.

Arguments:

InHandle [in] 		Handle of vpu decoder.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

## Decode

### VPU_DecDecodeBuf

VpuDecRetCode VPU_DecDecodeBuf (

VpuDecHandle 		InHandle,

VpuBufferNode		* pInData,

int			* pOutBufRetCode)

Description:

Function to deocde one frame. This function will see if there is decoded frame to be sent out, then process the input buffer of the current frame and decode it. The final decode state is saved in pOutBufRetCode.

Arguments:

InHandle [in/out]  	Handle of vpu decoder.

pInData [in]	Pointer to VpuBufferNode struct where the information of input buffer is stored.

pOutBufRetCode [out]	State of the input and output frame.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecGetOutputFrame

VpuDecRetCode VPU_DecGetOutputFrame (

VpuDecHandle 			InHandle,

VpuDecOutFrameInfo 		* pOutFrameInfo)

Description:

Function to get the next output frame information.

Arguments:

InHandle [in/out]  	Vpu decode object.

pOutFrameInfo [out]	Pointer to VpuDecOutFrameInfo struct where the output frame information is stored.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecGetConsumedFrameInfo

VpuDecRetCode VPU_DecGetConsumedFrameInfo (

VpuDecHandle 			InHandle,

VpuDecFrameLengthInfo	* pOutFrameInfo)

Description:

Function to get the information of frame buffers in vpu.

Arguments:

InHandle [in/out]  	Vpu decode object.

pOutFrameInfo [out]	Pointer to VpuDecFrameLengthInfo struct where the information of frame buffers in vpu is stored.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecOutFrameDisplayed

VpuDecRetCode VPU_DecOutFrameDisplayed (

VpuDecHandle 		InHandle,

VpuFrameBuffer	* pInFrameBuf)

Description:

Function to clear one display frame buffer.

Arguments:

InHandle [in/out]  	Handle of vpu decoder.

pInFrameBuf [out]	Pointer to VpuFrameBuffer struct which contains the information of frame buffer to be cleared.

Return value:

VpuDecRetCode   value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

## Memory Query and Free

### VPU_DecQueryMem

VpuDecRetCode VPU_DecQueryMem (

VpuMemInfo		* pOutMemInfo)

Description:

Funtion to query output physical and virtual memory information to further allocate.

Arguments:

pOutMemInfo [out] 	Pointer to VpuMemInfo struct where the output memory information is stored.

Return value:

VPU_DEC_RET_SUCCESS

### VPU_DecGetMem

VpuDecRetCode VPU_DecGetMem (

VpuMemDesc		* pInOutMem)

Description:

Function to allocate memory.

Arguments:

pInOutMem [in/out] 	Pointer to VpuMemDesc struct where the memory information is stored.

Return value:

VpuDecRetCode    value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecFreeMem

VpuDecRetCode VPU_DecFreeMem (

VpuMemDesc		* pInMem)

Description:

Function to free memory.

Arguments:

pInMem [in] 		Pointer to VpuMemDesc struct where the memory information is stored.

Return value:

VpuDecRetCode    value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

### VPU_DecRegisterFrameBuffer

VpuDecRetCode VPU_DecRegisterFrameBuffer (

VpuDecHandle 		InHandle,

VpuFrameBuffer 	* pInFrameBufArray,

int 			nNum)

Description:

Function to register frame buffer.

Arguments:

InHandle [in/out]	Handle of vpu encoder.

pInFrameBufArray [in]	Pointer to VpuFrameBuffer struct where the information of vpu frame buffer is stored.

nNum [in]		The number of vpu frame buffer count.

Return value:

VpuDecRetCode    value: VPU_DEC_RET_SUCCESS, VPU_DEC_RET_FAILURE…

# Encoder API Functions

## Encoder Open and Close

### VPU_EncGetVersionInfo

VpuEncRetCode VPU_EncGetVersionInfo (

VpuVersionInfo 		* pOutVerInfo)

Description:

Function to get the vpulib and firmware version.

Arguments

pOutVerInfo [out]        	Pointer to VpuVersionInfo struct where the output parameters will be stored.

Return value

VPU_ENC_RET_SUCCESS

### VPU_EncGetWrapperVersionInfo

VpuEncRetCode VPU_EncGetWrapperVersionInfo (

VpuWrapperVersionInfo 	* pOutVerInfo)

Description:

Function to get the vpu wrapper version.

Arguments

pOutVerInfo [out]   	Pointer to VpuWrapperVersionInfo struct where the output parameters will be stored.

Return value

VPU_ENC_RET_SUCCESS

### VPU_EncGetInitialInfo

VpuEncRetCode VPU_EncGetInitialInfo (

VpuEncHandle 		InHandle,

VpuEncInitInfo 		* pOutInitInfo)

Description:

Function to get the initial infomation.

Arguments:

InHandle [in]  	             Handle of vpu encoder.

pOutInitInfo [out] 	Pointer to VpuEncInitInfo struct where initial configuration parameters of the encoder will be stored.

Return value:

VPU_ENC_RET_SUCCESS

### VPU_EncConfig

VpuEncRetCode VPU_EncConfig (

VpuEncHandle 		InHandle,

VpuEncConfig 		InEncConf,

Void			* pInParam)

Desciption:

Function to set the corresponding parameters of encoder according to InEncConf type.

Arguments:

InEncConf [in] 		Type of configuration to be set.

pInParam [in]		Value that used to set the encoder according to InEncConf type.

InHandle [out]		Handle of vpu encoder.

Return value:

VpuEncRetCode    value: VPU_ENC_RET_SUCCESS, VPU_ENC_RET_FAILURE…

### VPU_EncOpen

VpuncRetCode VPU_EncOpen (

VpuEncHandle 		* pOutHandle,

VpuMemInfo		* pInMemInfo,

VpuEncOpenParam	* pInParam)

Description:

Function to open new vpu handle, called by function VPU_EncOpenSimp.

Arguments:

pInMemInfo [in]		Pointer to VpuMemInfo struct which contains the memory information.

pInParam [in]	Pointer to VpuEncOpenParam struct which contains input parameters of the encoder.

pOutHandle [out] 	Handle of vpu encoder.

Return value:

VpuEncRetCode    value: VPU_ENC_RET_SUCCESS, VPU_ENC_RET_FAILURE…

### VPU_EncOpenSimp

VpuEncRetCode VPU_EncOpenSimp (

VpuEncHandle 			* pOutHandle,

VpuMemInfo			* pInMemInfo,

VpuEncOpenParamSimp 	* pInParam)

Description:

Function to open new vpu handle which is realized in function VPU_EncOpen.

Arguments:

pInMemInfo [in]		Pointer to VpuMemInfo struct which contains the memory information.

pInParam [in]	Pointer to VpuEncOpenParam struct which contains input parameters of the encoder.

pOutHandle [out]	Handle of vpu encoder.

Return value:

VpuEncRetCode    value: VPU_ENC_RET_SUCCESS, VPU_ENC_RET_FAILURE…

### VPU_EncLoad

VpuEncRetCode VPU_EncLoad ()

Description:

Function to parse the log level, load vpu.

Arguments:

None.

Return value:

VPU_ENC_RET_SUCCESS

### VPU_EncUnLoad

VpuEncRetCode VPU_EncUnLoad ()

Description:

Function to unload vpu.

Arguments:

None.

Return value:

VPU_ENC_RET_SUCCESS

### VPU_EncReset

VpuEncRetCode VPU_EncReset (

VpuEncHandle 		InHandle)

Description:

Function to reset the handle of vpu. H1 encoder has no interface to reset.

Arguments:

InHandle [in] 		Handle of vpu encoder.

Return value:

VPU_ENC_RET_SUCCESS

### VPU_EncClose

VpuEncRetCode VPU_EncClose (

VpuEncHandle 		InHandle)

Description:

Function to close the encoder.

Arguments:

InHandle [in] 		Handle of vpu encoder.

Return value:

VPU_ENC_RET_SUCCESS

## Encode

### VPU_EncEncodeFrame

VpuEncRetCode VPU_EncEncodeFrame (

VpuEncHandle 		InHandle,

VpuEncEncParam	* pInOutParam)

Description:

Function to encode one frame. Before encoding the first frame, call function VPU_EncStartEncode to start stream. Otherwise, call function VPU_EncDoEncode.

Arguments:

InHandle [in/out] 	Handle of the vpu encoder.

pInOutParam [out] 	Pointer to VpuEncEncParam struct where input and output parameters are stored.

Return value:

VpuEncRetCode    value: VPU_ENC_RET_SUCCESS, VPU_ENC_RET_FAILURE…

## Memory Query and Free

### VPU_EncQueryMem

VpuEncRetCode VPU_EncQueryMem (

VpuMemInfo		* pOutMemInfo)

Description:

Funtion to query output physical and virtual memory information to further allocate.

Arguments:

pOutMemInfo [out] 	Pointer to VpuMemInfo struct where the output memory information is stored.

Return value:

VPU_ENC_RET_SUCCESS

### VPU_EncGetMem

VpuEncRetCode VPU_EncGetMem (

VpuMemDesc		* pInOutMem)

Description:

Function to allocate memory.

Arguments:

pInOutMem [in/out] 	Pointer to VpuMemDesc struct where the memory information is stored.

Return value:

VpuEncRetCode    value: VPU_ENC_RET_SUCCESS, VPU_ENC_RET_FAILURE…

### VPU_EncFreeMem

VpuEncRetCode VPU_EncFreeMem (

VpuMemDesc		* pInMem)

Description:

Function to free memory.

Arguments:

pInMem [in] 		Pointer to VpuMemDesc struct where the memory information is stored.

Return value:

VpuEncRetCode    value: VPU_ENC_RET_SUCCESS, VPU_ENC_RET_FAILURE…

### VPU_EncRegisterFrameBuffer

VpuEncRetCode VPU_EncRegisterFrameBuffer (

VpuEncHandle 		InHandle,

VpuFrameBuffer 	* pInFrameBufArray,

int 			nNum,

int 			nSrcStride)

Description:

Function to register frame buffer.

Arguments:

InHandle [in/out]	Handle of vpu encoder.

pInFrameBufArray [in]	Pointer to VpuFrameBuffer struct where the information of vpu frame buffer is stored.

nNum [in]		The number of minimum frame buffer count obtained from initial info.

nSrcStride [in]		The number of stride of plane 0.

Return value:

VPU_ENC_RET_SUCCESS

# API Calling Sequence

## This section gives the API calling sequence to further understand encoding and decoding process.

## Decoding calling sequence

## The block diagram of a typical decoding process is depicted in Figure 1. Main steps of decoding is described as follows:

Figure 1 Decoding process block diagram

- Load vpu.

VPU_DecLoad()

- Check vpulib and vpu wrapper version

VPU_DecGetVersionInfo()

VPU_DecGetWrapperVersionInfo()

- Query memory to hold for output buffer.

VPU_DecQueryMem()

Note: query memory information from vpu with VPU_EncQueryMem(), then vpu will allocate internal memory for it.

- Allocate memory for vpu.

VPU_DecGetMem()

- Open new vpu deocder handle

VPU_DecOpen()

Note: Configuration parameters will be set according to video type and decode handle will be created.

- Set decoding configuration for vpu decoder.

VPU_DecGetCapability()

VPU_DecConfig()

- Start to decode frame

VPU_DecDecodeBuf()

Note: function VPU_DecDecodeBuf() includes such steps.

- Scan whether there is decoded frame.If has decoded frame, the output state will has state VPU_DEC_OUTPUT_DIS to indicate there is a frame to be sent out.

- Process the current frame input data, the output state will has state VPU_DEC_INPUT_USED.

- Decode the current frame. If the input data is consumed and frame is decoded successfully, the output state will has state VPU_DEC_ONE_FRM_CONSUMED.

- After decoding, perform different actions according to output state.

VPU_DecGetOutputFrame()

VPU_DecGetConsumedFrameInfo()

VPU_DecFlushAll()

Note: the below is several actions can happen with output state pOutBufRetCode.

- If (pOutBufRetCode & VPU_DEC_ONE_FRM_CONSUMED) is not 0, then call VPU_DecGetConsumedFrameInfo() to get the frame information in vpu.

- If (pOutBufRetCode & VPU_DEC_OUTPUT_DIS) is not 0, then call VPU_DecGetOutputFrame() to get the output frame information.

- If (pOutBufRetCode & VPU_DEC_FLUSH) is not 0, then call VPU_DecFlushAll() to flush the decoder.

- Release frame buffer to vpu after display the frame

VPU_DecOutFrameDisplayed()

- Close vpu.

VPU_DecClose()

VPU_DecUnLoad()

- Release vpu memory.

VPU_DecFreeMem()

## Encoding calling sequence

## The block diagram of a typical encoding process is depicted in Figure 2. Main steps of encoding is described as follows:

Figure 2 Encoding process block diagram

- Load vpu.

VPU_EncLoad()

- Check vpulib and vpu wrapper version information.

VPU_EncGetVersionInfo()

VPU_EncGetWrapperVersionInfo()

- Query memory to hold for output buffer.

VPU_EncQueryMem()

Note: query memory information from vpu with VPU_EncQueryMem(), then vpu will allocate internal memory for it.

- Allocate memory for vpu.

VPU_EncGetMem()

- Open new vpu encoder handle

VPU_EncOpen()

VPU_EncOpenSimp()

Note: Configuration parameters will set be set according to encoded stream type and encode handle will be created.

- Set default configuration and get initial information of vpu encoder.

VPU_EncConfig()

VPU_EncGetInitialInfo()

- Allocate physical memory for input buffer and register frame buffer if needed.

VPU_EncRegisterFrameBuffer()

Note: H1 encoder don’t need to register frame buffer, so VPU_EncRegisterFrameBuffer() do nothing.

- Start to encode stream.

VPU_EncEncodeFrame()

Note: This is the stream production process, including stream start and stream encode. Encoder will first start stream and output codec data, then start to encode stream one frame by one frame.

- Close vpu

VPU_EncClose()

VPU_EncUnLoad()

- Release vpu memory

VPU_EncFreeMem()